Floating Static route  (this should be configured on all 3 routers)

in the global configuration

# ip route   <destination nid> <destination subnet mask> <next hop router's ip address>

<administrative distance more than 120 eg 121>

RIP :  (this should be configured on all 3 the routers>

router rip

version 2

network <nid of the network connected to LAN network>

network <nid of the network connected to another router>

no auto-summary

DHCP  on router (acting as DHCP server)

Ip dhcp excluded-address<ip address>

ip dhcp pool<poolname>

network <nid> <subnetmask>

default-router <ip address of the router>

Relay agent : this should be  configured on the relay agent router in the interface that is close to the dhcp client

#int <>

ip helper-address <ip address of the interface of the dhcp srv to which the relay agent connects>

Configuration on the switch:

Creating vlan

Switch(config)#vlan 10

Switch(config-vlan)#name <name of the vlan>

Switch(config-vlan)#int range fa0/1 - 10

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 10

Switch(config-if-range)#exit

Trunk link configuration

Switch(config)#INT FA0/24

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk native vlan <vlan number>

Show commands on switch:

Show vlan br : shows the vlan information in the switch

Show interfaces trunk: shows the configured trunk links on the lo switch and the vlans allowed through the trunk link

Show run: shows running configuration in the switch

Show commands on the router :

Show ip route : to see the router’s routing table

Show interfaces brief: gives brief info about the status of all the interfaces of the router

Show ip dhcp pool :  displays information about the configured DHCP pools

Show ip dhcp binding: view binding information for IP addresses and corresponding DHCP client hardware addresses.

Show run: shows running configuration in the router